HARDWARE SPECIFICATION

Flight Controller — Prototype v1

| Parameter | Value |
| --- | --- |
| Document ID | HW-SPEC-PROTO-001 |
| Version | 1.0 — Draft |
| Date | February 2026 |
| Parent document | SYS-REQ-001 |
| Status | Draft |
| Certification | Not certified — internal validation prototype only |

# 1. Purpose

This document specifies the hardware requirements for the design and manufacture of the Flight Controller (FC) Prototype v1. It is intended for the hardware engineer responsible for PCB design, layout, and assembly.

The prototype serves to validate hardware choices prior to the certified DAL-B production design. It is not subject to DO-254 certification at this stage.

# 2. System Architecture

The FC system consists of three separate PCBs interconnected via board-to-board connectors:

| Board | PCB Size | Role |
| --- | --- | --- |
| FCU (Main Board) | 100 × 100 mm | MCU, inertial sensors, power interfaces, communication |
| GNSS + Compass Module | External module | u-blox ZOE-M8Q + AK09918C |
| Remote Control Module | External module | ExpressLRS SX1280 + ESP32-C3 |

- GNSS and RC modules connect to the FCU via board-to-board stacking connectors (left and right sides of the FCU)

- All three PCBs are 4-layer

## 2.1 Block Diagram

Figure 1 — FC System Block Diagram

## 2.2 Mechanical Layout

Figure 2 — FCU Board Layout (100 × 100 mm octagonal, modules on left and right sides)

# 3. Fixed Components

The following critical components are imposed. No substitution without prior validation.

## 3.1 FCU — Main Board

Microcontroller

| Parameter | Value |
| --- | --- |
| MCU | ST32H753ZI |
| Architecture | ARM Cortex-M7 @ 480 MHz — hardware double-precision FPU |
| Justification | 16D EKF @ 1 kHz — SYS-REQ-F-010, SYS-REQ-F-013 |

Inertial Sensors — Redundancy Required (SYS-REQ-F-001)

| Reference | Component | Bus | Rate | Notes |
| --- | --- | --- | --- | --- |
| IMU 1 | BMI088 | SPI | ≥ 1 kHz | Gyro + Accel |
| IMU 2 | ICM-42688-P | SPI | ≥ 1 kHz | Gyro + Accel |
| Barometer 1 | BMP390 | SPI | ≥ 50 Hz |  |
| Barometer 2 | DPS368 | SPI | ≥ 50 Hz |  |

IMPORTANT: Both IMUs and both barometers must be mounted on vibration isolation pads (soft-mount).

Distance Sensor

| Reference | Component | Bus | Rate |
| --- | --- | --- | --- |
| Lidar | TF02 Pro | UART | ≥ 50 Hz — SYS-REQ-F-001 |

## 3.2 GNSS + Compass Module (external board)

| Reference | Component | Interface to FCU | Rate |
| --- | --- | --- | --- |
| GNSS | u-blox ZOE-M8Q | UART | ≥ 5 Hz — SYS-REQ-F-001 |
| Magnetometer | AK09918C | SPI | ≥ 50 Hz — SYS-REQ-F-001 |

- External GNSS antenna via U.FL or SMA connector

- Module positioned on the right/left side of the FCU (see Figure 2)

## 3.3 Remote Control Module (external board)

| Reference | Component | Interface to FCU | Notes |
| --- | --- | --- | --- |
| RF Transceiver | SX1280 | SPI (via ESP32-C3) | ExpressLRS — SYS-REQ-IF-001 |
| RF MCU | ESP32-C3 | UART CRSF to FCU | Primary CRSF protocol |

- FCU interfaces: UART 1 (CRSF primary) + UART 2 (secondary)

- Module positioned on the left/right side of the FCU (see Figure 2)

# 4. Power Supply

The FCU has no direct access to the battery. All power rails are supplied by the propulsion board via dedicated JST-PH 2-pin connectors (one per rail). Maximum current per rail: 2A.

| Rail | Voltage | Max Current | Connector | Usage |
| --- | --- | --- | --- | --- |
| 3.3V Digital | 3.3V regulated | 2A | JST-PH 2-pin | MCU, digital sensors, logic |
| 3.3V Analog | 3.3V regulated | 2A | JST-PH 2-pin | Analog sensors, MCU ADC — filtered, isolated from digital rail |
| 12V Regulated | 12V regulated | 2A | JST-PH 2-pin | Buzzer, peripherals |

NOTE: The FCU does not include any DC-DC converters or LDOs for these rails. The three JST-PH connectors are the sole power inputs from the propulsion board. Current monitoring is not available on the FCU side.

Battery voltage and current monitoring (SYS-REQ-PWR-001/002) are provided by the propulsion board and transmitted to the FCU via CAN bus.

# 5. PCB Specifications

| Parameter | Specification |
| --- | --- |
| FCU dimensions | 100 × 100 mm — octagonal shape (chamfered corners) |
| Module dimensions | Not specified |
| Number of layers | 4 layers — all boards |
| Mounting holes FCU | 4× M3 — one at each corner |
| Module mounting | Board-to-board stacking connectors, lateral overhang |

# 6. Required Interfaces

## 6.1 Sensor Interfaces (FCU)

| Bus | Qty | Assignment | Notes |
| --- | --- | --- | --- |
| SPI | 4× | IMU1, IMU2, Baro1, Baro2 | Dedicated CS per sensor |
| UART | 1× | Lidar TF02 Pro | Verify TF02 operating voltage (5V or 3.3V) |

## 6.2 Motor Control Interface (FCU)

| Interface | Qty | Usage |
| --- | --- | --- |
| FD-CAN | 2× | Motor control (CAN Bus) — SYS-REQ-F-020 |

NOTE: FD-CAN is selected for motor control. ESCs are CAN-compatible.

## 6.3 Additional Communication Interfaces (FCU)

| Interface | Qty | Usage |
| --- | --- | --- |
| UART | 2× | Additional interfaces (e.g. telemetry, expansion) |
| I3C/I2C | 1× | Additional interface |
| GPIO | 4× | General purpose |

## 6.4 System Interfaces (FCU)

| Interface | Qty | Usage |
| --- | --- | --- |
| USB-C | 1× | System monitoring (USB 2.0) — SYS-REQ-NF-030 |
| SWD | 1× | MCU programming and debug |
| UART debug | 1× | MCU serial debug |
| SD Card | 1× | Flight data logging — SYS-REQ-IF-011 (SPI bus) |

## 6.5 Board-to-Board Interfaces (FCU ↔ Modules)

| Connection | Direction | Signals |
| --- | --- | --- |
| FCU ↔ GNSS+Compass | Board-to-board | UART, SPI, 3.3V Digital, GND |
| FCU ↔ RC Module | Board-to-board | UART1 (CRSF), UART2, 3.3V Digital, GND |

Connector reference, pitch, and stack height are left to the hardware engineer's judgment based on mechanical constraints and availability.

# 7. Visual and Audio Indicators

| Component | Specification | SYS-REQ |
| --- | --- | --- |
| Red LED | 1× red LED — critical state | SYS-REQ-ARM-003 |
| Blue LED | 1× blue LED — info state | SYS-REQ-ARM-003 |
| Green LED | 1× green LED — armed / OK | SYS-REQ-ARM-003 |
| RGB LED | 1× RGB LED — general status | SYS-REQ-ARM-003 |
| Passive Buzzer | 100 dB at 30 cm | SYS-REQ-ARM-003 |

# 8. Physical Controls

| Element | Specification |
| --- | --- |
| RESET button | 1× push button — MCU reset |
| General purpose btn | 1× push button — configurable (e.g. manual arming) |

# 9. Hardware Security

| Element | Specification |
| --- | --- |
| External watchdog | 1× hardware watchdog — 500 ms timeout — SYS-REQ-NF-023 |
| Justification | System reset if not refreshed by MCU — protects against software lockup |

# 10. Timing Requirements

These constraints are critical for the flight control loop. The hardware engineer must ensure all interfaces support the following data rates.

| Loop / Sensor | Rate / Period | Reference |
| --- | --- | --- |
| IMU (gyro + accel) | ≥ 1 kHz | SYS-REQ-F-013, SYS-REQ-NF-010 |
| Fast control loop (rate) | 1 ms — max jitter 0.2 ms | SYS-REQ-NF-010 |
| Medium control (attitude) | 5 ms — max jitter 0.5 ms | SYS-REQ-NF-010 |
| Barometer | ≥ 50 Hz | SYS-REQ-F-001 |
| GNSS | ≥ 5 Hz | SYS-REQ-F-001 |
| Telemetry / Logging | ≥ 1 Hz / ≥ 10 Hz | SYS-REQ-IF-005, SYS-REQ-IF-012 |

# 11. Deliverables

| # | Deliverable | Details |
| --- | --- | --- |
| 1 | Schematic | PDF + source file (KiCad) |
| 2 | PCB layout | Gerber files + drill files |
| 3 | BOM | Manufacturer references, quantities, suppliers |
| 4 | Design notes | Key decisions, known limitations, simulation results |

# 12. Acceptance Criteria

The prototype is considered acceptable when all of the following checks pass:

| # | Criterion | Reference |
| --- | --- | --- |
| 1 | MCU boots successfully in < 5 seconds | SYS-REQ-SM-002 |
| 2 | IMU 1 (BMI088) readable at 1 kHz via SPI | SYS-REQ-F-001 |
| 3 | IMU 2 (ICM-42688-P) readable at 1 kHz via SPI | SYS-REQ-F-001 |
| 4 | Barometer 1 (BMP390) and Barometer 2 (DPS368) readable at 50 Hz | SYS-REQ-F-001 |
| 5 | GNSS (ZOE-M8Q) — fix acquired, data @ 5 Hz | SYS-REQ-F-001 |
| 6 | Magnetometer (AK09918C) readable at 50 Hz | SYS-REQ-F-001 |
| 7 | Lidar (TF02 Pro) readable via UART at 50 Hz | SYS-REQ-F-001 |
| 8 | SD Card — read/write functional | SYS-REQ-IF-011 |
| 9 | FD-CAN — frame transmitted and received on both buses | SYS-REQ-F-020 |
| 10 | USB-C — serial communication active | SYS-REQ-NF-030 |
| 11 | External watchdog — system reset triggered if not refreshed in 500ms | SYS-REQ-NF-023 |
| 12 | All three power rails stable under load (3.3V dig, 3.3V ana, 12V) | SYS-REQ-PWR-001 |
| 13 | RGB LED + red/blue/green LEDs — all functional | SYS-REQ-ARM-003 |
| 14 | Buzzer — tone audible at 30 cm | SYS-REQ-ARM-003 |
| 15 | IMU loop jitter < 0.2 ms (measured via oscilloscope or GPIO toggle) | SYS-REQ-NF-010 |

# 13. Design Freedom

The following design decisions are left to the hardware engineer's judgment:

- Board-to-board connector selection (pitch, stack height, manufacturer reference)

- External connectors type and reference (JST-GH recommended but not mandatory)

- Component placement and PCB routing

- Analog power rail filtering strategy (EMI filters, ferrite beads, decoupling)

- Passive component footprints

# Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | Feb. 2026 | BASSINGA Brisel Andresse | Initial release |